#### Repair pause

##### Repair paused

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | R | E | P | A | I | R |  | B | U | S | Y |  |  |  |  |
| 3 |  |  | H | i | t |  |  | X |  |  | t | o |  | P | A | U | S | E |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

When X is hit during an active repair, the screen above appears. This is to suppress accidental keyboard actions. If a pause is requested the user must press X a second time. This screen will remain on display for one second. If the X-button is not pressed within this time, the system will return to the “repair in progress” screen.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

Pausing the repair process automatically turns down the voltage output of all rows to 0 volts. It can take some time to unload all capacities on the rows, so the system will keep waiting until all rows have a voltage below the safety threshold. The display will notify about this waiting time with the following screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | W | A | I | T | I | N | G |  | F | O | R |  |  |  |  |  |
| 4 |  |  |  |  |  | P | O | W | E | R |  | D | O | W | N |  |  |  |  |  |

When treatment is paused, the display will change its output to:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | ! | ! | P | A | U | S | E | ! | ! |  |  |  |  |  |  |
| 3 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | e |
| 4 |  | H | i | t |  |  | X |  |  | t | o |  | R | E | S | E | T |  |  |  |

| ENT | Continue the current repair |
| --- | --- |
| X | Reset/quit the treatment and return to the tube repair menu |
| others | No function |