Photonis 0387 HSB MRTS User Manual. Description: User manual for HSB module repair test set; Doc No.:; PHO-0387-U0-3-1. Revision:; 001. Date:; 23-2-2018.

|  | Name | Date | Signature |
| --- | --- | --- | --- |
| Author | T. Lantinga | 23-2-2018 |  |
| Reviewed |  |  |  |
| Approved |  |  |  |

* The approval box may be extended as necessary.

No part of this document may be reproduced in any form or by any means, without the prior written permission of the respective owners. All trademarks are property of their respective owners.

Document History

| Version | Date | Status | Description |
| --- | --- | --- | --- |
| 001 | 23-2-2018 | draft | Initial draft |

Table of Contents

1	General	6

1.1	Symbol and warnings	6

1.2	Disposal of equipment	6

1.3	Intended usage	6

1.4	Safety	6

1.4.1	Safety notes	6

1.4.2	Responsibility of the operator	7

1.4.3	Responsibility of the administrator	7

1.5	Technical data	7

2	Installation	8

2.1	Placement	8

2.2	DUT connection	8

2.3	Auxiliary mains socket	8

3	Operation	9

3.1	Front panel control	9

3.2	Drawer control	9

3.3	Anode drawer	10

3.3.1	Start screen	10

3.3.2	Main menu	10

3.3.3	Opening the cover	10

3.3.4	Tube repair menu	10

3.3.4.1	Select preset menu	11

3.3.4.2	Preset preview screen	11

3.3.4.3	Start repair menu	11

3.3.4.3.1	Error messages based on safety	11

3.3.4.3.2	Repair in progress	12

3.3.4.3.3	Repair Finished	12

3.3.5	Repair pause	12

3.3.5.1	Repair paused	12

3.3.6	Administrator Menu	13

3.3.6.1	PIN protection	13

3.3.6.1.1	PIN insertion screen	13

3.3.6.1.2	PIN accepted	14

3.3.6.1.3	PIN denied	14

3.3.6.2	Administrator main menu	14

3.3.6.3	Change PIN menu	15

3.3.6.3.1	Insert new Pin	15

3.3.6.3.2	New Pin OK	15

3.3.6.3.3	New Pin failed	16

3.3.6.4	I/O control menu	16

3.3.6.4.1	Read interlock	16

3.3.6.4.2	Solenoids	17

3.3.6.4.3	Get voltage in	17

3.3.6.4.4	Set voltage out	17

3.3.6.5	Preset configuration	18

3.3.6.6	PID constants	20

3.3.6.7	Information and version screen	20

3.3.7	Calibration menu	20

3.3.7.1	PIN protection	20

3.3.7.1.1	PIN insertion screen	20

3.3.7.1.2	PIN accepted	21

3.3.7.1.3	PIN denied	21

3.3.7.2	Calibration main menu	21

3.3.7.3	Setpoint check menu	22

3.3.7.3.1	Low setpoint check screen	22

3.3.7.3.2	High setpoint check screen	23

3.4	Cathode / MCP drawer	23

3.4.1	Start screen	23

3.4.2	Main menu	23

3.4.3	Opening the cover	24

3.4.4	Tube repair menu	24

3.4.4.1	Select preset menu	24

3.4.4.2	Preset preview screen	25

3.4.4.3	Start repair menu	25

3.4.4.3.1	Error messages based on safety	25

3.4.4.3.2	Repair in progress	25

3.4.4.3.3	Repair Finished	26

3.4.5	Repair pause	26

3.4.5.1	Repair paused	26

3.4.6	Administrator Menu	27

3.4.6.1	PIN protection	27

3.4.6.1.1	PIN insertion screen	27

3.4.6.1.2	PIN accepted	27

3.4.6.1.3	PIN denied	28

3.4.6.2	Administrator main menu	28

3.4.6.3	Change PIN menu	29

3.4.6.3.1	Insert new Pin	29

3.4.6.3.2	New Pin OK	29

3.4.6.3.3	New Pin failed	30

3.4.6.4	I/O control menu	30

3.4.6.4.1	Read interlock	30

3.4.6.4.2	Solenoids	31

3.4.6.4.3	Get voltage in	31

3.4.6.4.4	Set voltage out	31

3.4.6.5	Preset configuration	32

3.4.6.6	PID Constants	34

3.4.6.7	Information and version screen	34

3.4.7	Calibration menu	35

3.4.7.1	PIN protection	35

3.4.7.1.1	PIN insertion screen	35

3.4.7.1.2	PIN accepted	35

3.4.7.1.3	PIN denied	35

3.4.7.2	Calibration main menu	36

3.4.7.3	Cathode/MCP selection menu	36

3.4.7.4	Setpoint check menu	36

3.4.7.4.1	Low setpoint check screen	37

3.4.7.4.2	High setpoint check screen	37

3.5	Tesla drawer	38

3.5.1	Start screen	38

3.5.2	Main menu	38

3.5.3	Opening the cover	38

3.5.4	Tube repair menu	38

3.5.4.1	Select preset menu	39

3.5.4.2	Preset preview screen	39

3.5.4.3	Start repair menu	39

3.5.4.3.1	Error messages based on safety	39

3.5.4.3.2	Repair in progress	40

3.5.4.3.3	Repair finished	40

3.5.5	Repair pause	41

3.5.5.1	Repair paused	41

3.5.6	Administrator Menu	41

3.5.6.1	PIN protection	41

3.5.6.1.1	PIN insertion screen	41

3.5.6.1.2	PIN accepted	42

3.5.6.1.3	PIN denied	42

3.5.6.2	Administrator main menu	42

3.5.6.3	Change PIN menu	43

3.5.6.3.1	Insert new Pin	43

3.5.6.3.2	New Pin OK	43

3.5.6.3.3	New Pin failed	44

3.5.6.4	I/O control menu	44

3.5.6.4.1	Read interlock	44

3.5.6.4.2	Solenoids	45

3.5.6.4.3	Get voltage in	45

3.5.6.4.4	Set voltage out	45

3.5.6.4.5	Teslagun relais	46

3.5.6.5	Preset configuration	46

3.5.6.6	PID constraints	48

3.5.6.7	Information and version screen	48

3.5.7	Calibration menu	48

3.5.7.1	PIN protection	48

3.5.7.1.1	PIN insertion screen	48

3.5.7.1.2	PIN accepted	49

3.5.7.1.3	PIN denied	49

3.5.7.2	Calibration main menu	50

3.5.7.2.1	Setpoint check menu	50

3.6	Error messages	51

3.6.1	Cover open	51

4	Contact and support	52

4.1	Maintenance and cleaning	52

4.2	Repairs	52

4.3	Replacing a defect mains fuse	52

4.4	Contact	52

# General

## Symbol and warnings

Symbol for general safety notices

Symbol for a life-threatening danger

## Disposal of equipment

A piece of equipment which is intended for disposal must, according to European laws and regulations (WEEE) be returned to Photonis for scrapping, unless the person operating the piece of equipment or another, delegated person is conducting the disposal. Our equipment falls under these regulations and is accordingly marked with the following symbol:

## Intended usage

The equipment includes 3 repair drawers, each with a unique repair process.

The equipment is intended to be used only for repairing defective image intensifier tubes, in the corresponding repair drawer.

•	Claims of any sort due to damage caused by non-intended usage will not be accepted.

•	All damage caused by non-intended usage is solely the responsibility of the operator.

## Safety

### Safety notes

- The equipment must only be used as intended.

- Do not insert any object, particularly metallic, in the drawers.

- Avoid any use of liquids near the equipment. Protect the device from wet, damp and condensation.

- The Auxiliary mains socket is only to be used in conjunction with the Tesla drawer.

Danger for unqualified users

Improper operation can cause person or object damage.

To guarantee safety when using the device, it is essential that only persons operate the device who are fully acquainted and trained in the required safety measures to be taken when working with dangerous electrical voltages.

### Responsibility of the operator

The operator has been properly instructed in their tasks and the attendant dangers. The operator is only allowed to execute the repair process.

### Responsibility of the administrator

The administrator is able through training, knowledge and experience as well as knowledge of

the specific details to carry out all the required tasks, identify dangers and avoid personal and other risks.

- The tasks of an administrator include:

- Executing repair process

- Access to administrator menu and thereby able to:

  - Change PIN

  - Control individual I/O

  - Change the preset configurations

  - Change the PID constants

- Access to calibration menu and thereby able to:

  - Execute calibration process

## Technical data

| Mains supply |  |
| --- | --- |
| Rated voltage | 220-240 VAC |
| Frequency | 47-63 Hz |
| Power dissipation | 80 Watt |
| Fuse rating | 3.15A |
| Fuse type | Slow |

| Operating Conditions |  |
| --- | --- |
| Safety class | Class I |
| Operating temperature | 21 °C ± 2°C |
| Operating humidity | 45% ± 10% |
| Max. installation height | 2000 m |
| IP class | IP20 |

| DC output |  |
| --- | --- |
| Installation class |  |
| Safety class | Class I |
| Operating temperature |  |
| Operating humidity |  |
| Max. installation height | 2000 m |
| IP class | IP20 |

| Auxiliary mains socket |  |
| --- | --- |
| Rated voltage |  |
| Frequency | Class I |
| Max. power consumption |  |

# Installation

## Placement

Info Photonis

## DUT connection

Photos

## Auxiliary mains socket

The Auxiliary mains socket is used during the repair process in the Tesla drawer. It’s intended for a high voltage spark tester capable of generating an output voltage that is adjustable between at least 10kV to 55kV. The spark tester must not exceed the rating of the Auxiliary mains socket as mentioned in chapter 1.5 Technical data. The spark tester must comply with the applicable European laws and regulations, regarding safety and EMC.

# Operation

## Front panel control

On the front top side there is a Control panel. It has the following functions:

- Main switch

Use this switch to enable power to the device. All drawers are powered simultaneously with this switch.

- Main On

White indicator light, indicating if the equipment is connected to the mains supply.

- Emergency switch

In case of an emergency, press this switch to interrupt the mains power.

Pull and rotate the emergency switch to enable the mains power again.

- Auxiliary mains socket

Use the Auxiliary mains socket for a suitable spark tester, which will be used during the Tesla repair process.

- Auxiliary enable switch

During the Tesla repair process, press this switch to enable mains power to the Auxiliary mains socket. This switch is in series with a safety feature implemented in de Tesla drawer. The operator needs both hands to operate the spark tester; one hand on this enable switch and the other hand on the spark tester.

## Drawer control

Interaction between user and system occurs via a display and a keypad which are part of the HSB module repair test set.

Within this document the display content is related to by a table. Below represents the display seen from left to right and up to down as a matrix of 4x20.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

See the table below for an example.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | c | h | e | d | u | l | e | : |  | M | o | n | . | - |  | F | r | i | . |
| 2 | O | p | t | i | o | n | s |  | P | r | e | s | e | t | : |  | A | 2 | 1 |  |
| 3 | A | u | t | o | - | o | f | f | : |  | N | O |  |  |  |  |  |  |  |  |
| 4 | A | c | c | e | s | s | : |  | P | u | b | l | i | c |  |  |  |  |  |  |

The system contains a keypad which represents the only possibility for the user to interact with the several functions of the system.

The keypad consists of a matrix of four by four keys, making it 16 keys totally. They are labelled as follows:

| 0-9 | Numeric keys 0 - 9 |
| --- | --- |
| ← | Arrow left |
| ↑ | Arrow up |
| → | Arrow right |
| ↓ | Arrow down |
| ENT | Enter |
| X | Escape |

## Anode drawer

### Start screen

After powering the system, the display will become active and print out a start message, containing the type of repair process and the software version number.

A full start message on display, containing the system version number, will look like:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | n | o | d | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | S | W |  | V | . |  | 1 | . | 0 | . | 0 | . | 0 |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

There is no user-action needed at this point – the system will switch to the main menu as soon as the system is fully booted and functional.

### Main menu

After booting, the system switches to the main menu.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | n | o | d | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |
| 3 |  |  | 2 | . | A | d | m | i | n | i | s | t | r | a | t | o | r |  |  |  |
| 4 |  |  | 3 | . | C | a | l | i | b | r | a | t | i | o | n |  |  |  |  |  |

Note: The item 2 and 3 are only accessible for the administrator and therefor PIN protected.

| 1-3 | Select item 1-3 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| others | No function |

### Opening the cover

When the system is in main menu, press and hold the 0 key to unlock the cover lock.

### Tube repair menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | S | e | l | e | c | t |  | p | r | e | s | e | t |  |  |  |
| 3 |  |  | 2 | . | S | t | a | r | t |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 1-2 | Select item 1-2 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

#### Select preset menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | l | e | c | t |  | p | r | e | s | e | t |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | P | r | e | s | e | t |  | 1 |  |  |  |  |  |  |  |  |
| 3 |  |  | 2 | . | P | r | e | s | e | t |  | 2 |  |  |  |  |  |  |  |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | 3 |  |  |  |  |  |  |  |  |

There are a total of 9 presets.

| 1-9 | Select item 1-9 respectively and return to tube repair menu |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| → | Go to preset preview menu of the current currently under the cursor |
| ENT | Select preset currently under the cursor and return to tube repair menu |
| X | Leave current menu level and return to tube repair menu |
| others | No function |

Note: The system will always assume preset1 selected when no other selection is done.

#### Preset preview screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | X |  | i | n | f | o |  |  |  |  |  |  |  |
| 2 | S | t | a | r | t | : |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 3 | T | i | m | e | : |  |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 4 | V | o | l | t | : |  |  | - | 1 | 2 | 3 | 4 | m |  | - | 1 | 2 | 3 | 4 | m |

Where ‘X’ is the number of the preset that has been selected for preview.

| ← | Go back to select preset menu |
| --- | --- |
| X | Go back to select preset menu |
| others | No function |

#### Start repair menu

##### Error messages based on safety

Before starting the repair treatment, it will be verified that the cover of the drawer is closed correctly. For safety reason the cover must be closed before the repair treatment can be started.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  | ! | ! | E | R | R | O | R | ! | ! |  |  |  |  |  |  |
| 2 |  |  |  |  | C | O | V | E | R |  | O | P | E | N |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | H | i | t |  | X |  | t | o |  | c | o | n | t | i | n | u | e |  |

| X | Acknowledge error and return to main menu |
| --- | --- |
| others | No function |

Note: it is not sufficient to close the cover. The system will only react to the X key and return to the main menu. Closing the cover is, however, mandatory before starting a new repair.

##### Repair in progress

When all conditions are met, the repair treatment starts. The display now acts as feedback

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X |  |  | h | h | : | m | m | : | s | s |  | t | o | t | a | l |  |  |  |
| 2 |  |  |  | h | h | : | m | m | : | s | s |  | r | e | m | a | i | n |  |  |
| 3 |  |  | R | 1 |  |  |  |  |  | R | 2 |  |  |  |  |  | R | 3 |  |  |
| 4 | - | 1 | 2 | 3 | 4 | V |  | - | 1 | 2 | 3 | 4 | V |  | - | 1 | 2 | 3 | 4 | V |

The number in row1/column1 represents the selected preset number.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

##### Repair Finished

The standard screen as soon as the repair process finishes looks like

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

| ENT | Return to main menu |
| --- | --- |
| others | No function |

In case that one or more errors occurred, the following screen will be shown indicating the failing row number(s).

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  | E | r | r | o | r |  | i | n |  | r | o | w |  |  |  |  |
| 3 |  |  |  |  |  |  |  | 1 |  | 2 |  | 3 |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

Where the numbers in third row show the row(s) that failed.

| ENT | Return to main menu |
| --- | --- |
| others | No function |

### Repair pause

#### Repair paused

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | R | E | P | A | I | R |  | B | U | S | Y |  |  |  |  |
| 3 |  |  | H | i | t |  |  | X |  |  | t | o |  | P | A | U | S | E |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

When X is hit during an active repair, the screen above appears. This is to suppress accidental keyboard actions. If a pause is requested the user must press X a second time. This screen will remain on display for one second. If the X-button is not pressed within this time, the system will return to the “repair in progress” screen.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

Pausing the repair process automatically turns down the voltage output of all rows to 0 volts. It can take some time to unload all capacities on the rows, so the system will keep waiting until the voltage of all rows is decreased to a safe level. The display will notify about this waiting time with the following screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | W | A | I | T | I | N | G |  | F | O | R |  |  |  |  |  |
| 4 |  |  |  |  |  | P | O | W | E | R |  | D | O | W | N |  |  |  |  |  |

When treatment is paused, the display will change its output to:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | ! | ! | P | A | U | S | E | ! | ! |  |  |  |  |  |  |
| 3 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | e |
| 4 |  | H | i | t |  |  | X |  |  | t | o |  | R | E | S | E | T |  |  |  |

| ENT | Continue the current repair |
| --- | --- |
| X | Reset/quit the treatment and return to the tube repair menu |
| others | No function |

After each pause the remaining repair time increases with 5 minutes.

### Administrator Menu

#### PIN protection

##### PIN insertion screen

The operator menu is PIN protected. Prior to entering, the user must enter the PIN. The default PIN is 0123. It is advised that the administrator changes this PIN as soon as possible.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | I | n | s | e | r | t |  | P | I | N |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | - | - | - | - |  |  |  |  |  |  |  |  |

| 0-9 | Insert number |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| X | Return to main menu |
| others | No function |

##### PIN accepted

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | P | I | N |  | O | K |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the operator menu will be entered.

##### PIN denied

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | P | I | N |  | D | E | N | I | E | D |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after one second and the system will return to the main menu.

#### Administrator main menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | d | m | i | n | i | s | t | r | a | t | i | o | n |  |  |  |  |  |  |
| 2 | > |  | 1 | . | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |
| 3 |  |  | 2 | . | I | / | O |  | c | o | n | t | r | o | l |  |  |  |  |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | c | o | n | f | i | g |  |  |  |

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | . | P | I | D |  | c | o | n | s | t | a | n | t | S |  |  |  |
|  |  |  | 5 | . | I | n | f | o |  | & |  | V | e | r | s | i | o | n |  |  |

| 1-5 | Select item 1-5 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

#### Change PIN menu

##### Insert new Pin

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | N | e | w |  | P | I | N | : |  |  |  | - | - | - | - |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave current menu level and return to administrator menu without changing the PIN |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| 0-9 | Insert number |
| others | No function |

When the 4-digit PIN has been entered, the screen automatically changes to:

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | N | e | w |  | P | I | N | : |  |  |  | * | * | * | * |  |  |
| 4 |  | R | e | p | e | a | t |  | P | I | N | : |  |  |  | - | - | - | - |  |  |

| X | Leave current menu level and return to administrator main menu without changing the PIN |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| 0-9 | Insert number |
| others | No function |

When the 4-digit has been entered a second time, the system will verify that both entered PINs are equal.

##### New Pin OK

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | N | e | w |  | P | I | N |  | a | c | c | e | p | t | e | d |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the administrator main menu will appear again.

##### New Pin failed

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | N | e | w |  | P | I | N |  | d | e | n | i | e | d |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the administrator main menu will appear again.

#### I/O control menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | / | O |  | c | o | n | t | r | o | l |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | R | e | a | d |  | i | n | t | e | r | l | o | c | k |  |  |
| 3 |  |  | 2 | . | S | o | l | e | n | o | i | d | S |  |  |  |  |  |  |  |
| 4 |  |  | 3 | . | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | . | S | e | t |  | v | o | l | t | a | g | e |  | o | u | t |  |

| 1-4 | Select item 1-4 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to administrator menu |
| others | No function |

##### Read interlock

Reading the interlock status is a constant action, meaning that the screen will immediately update its content with the interlock status.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | C | o | v | e | r |  | i | n | t | e | r | l | o | c | k |  | i | s |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

If interlock is open:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  |  | o | p | e | n |  |  |  |  |  |  |  |  |

If interlock is closed:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  | c | l | o | s | e | d |  |  |  |  |  |  |  |

| X | Leave current menu level and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Solenoids

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | o | l | e | n | o | i | d | s |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | H | o | l | d |  | 0 |  | t | o |  | t | o | g | g | l | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 0 | Press: activate solenoids (open cover) Release: de-activate solenoids (seal cover) |
| --- | --- |
| X | Leave current menu level and return to administrator I/O menu |
| others | No function |

##### Get voltage in

The screen to read the voltage input is a read-only screen, meaning that no user action is accepted (other than leaving the menu).

The screen shows the currently read voltage on all three rows in both voltage (left value) and the corresponding hexadecimal value of the DA converter.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | 1 | 2 | 3 | 4 | 5 | V | ¦ |  | A | B | C | D | x |  |
| 3 | R | o | w | 2 | : |  | 1 | 2 | 3 | 4 | 5 | V | ¦ |  | A | B | C | D | x |  |
| 4 | R | o | w | 3 | : |  | 1 | 2 | 3 | 4 | 5 | V | ¦ |  | A | B | C | D | x |  |

| X | Leave screen and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Set voltage out

The screen to set the voltage output enables the administrator to apply a voltage to the three rows.

The cursor is put at the first row when entering the menu.

The screen shows two values per row. The left value is the value that the administrator inserts and applies to the DA converter of that specific row. The right value of that row is the actual value that is read back from the AD converter of that same row.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | t |  | v | o | l | t | a | g | e |  | o | u | t |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | 1 | 2 | 3 | 4 | 5 | V | ¦ |  | 1 | 2 | 3 | 4 | 5 | V |
| 3 | R | o | w | 2 | : |  | 1 | 2 | 3 | 4 | 5 | V | ¦ |  | 1 | 2 | 3 | 4 | 5 | V |
| 4 | R | o | w | 3 | : |  | 1 | 2 | 3 | 4 | 5 | V | ¦ |  | 1 | 2 | 3 | 4 | 5 | V |

| 0-9 | Used for value insertion |
| --- | --- |
| ↑ | Move cursor up one row |
| ↓ | Move cursor down one row |
| ← | Backspace – delete last entered digit |
| ENT | Accept inserted value and apply to DA converter of that row |
| X | Leave screen and return to administrator I/O menu |
| others | No function |

Electrocution hazard

Set voltage out can be executed while the lid is open. This means the user can access the DUT connections while the high voltage is still present.

Leaving the set voltage out menu causes the following screen to appear.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | A | l | l |  | r | o | w | s |  | r | e | s | e | t |  | t | o |  | 0 | V |
| 4 |  | P | r | e | s | s |  | X |  | t | o |  | c | o | n | t | i | n | u | e |

| X | Leave screen and return to administrator I/O menu |
| --- | --- |
| others | No function |

#### Preset configuration

The repair process follows a configurable preset. There are two types of presets, a one-stage and a two-stage repair preset. See the following images for an explanation of the presets.

Figure 1: One stage repair preset

Figure 2: Two-stage repair preset

| Parameter | Description |
| --- | --- |
| Start | Ramp-up time (minutes) |
| Time | Repair duration (minutes) |
| Volt | Repair voltage (Volt) |

Select a preset to configure.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | c | o | n | f | i | g |  |  |  |  |  |  |  |
| 2 |  |  | 1 | . | P | r | e | s | e | t |  | 1 |  | c | o | n | f | i | g |  |
| 3 |  |  | 2 | . | P | r | e | s | e | t |  | 2 |  | c | o | n | f | i | g |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | 3 |  | c | o | n | f | i | g |  |

The screen continues until item “9.Preset 9 config”.

| 1-9 | Select item 1-9 respectively |
| --- | --- |
| ↑ | Move cursor up one row |
| ↓ | Move cursor down one row |
| ENT | Select current preset under the cursor |
| X | Leave screen and return to Cathode/MCP selection menu |
| others | No function |

The first six presets only contain one stage, while preset seven, eight and nine contain two repair stages. The screen underneath represents a preset with two stages. For a single-stage preset, the second column with values is not available.

When entering this screen, the cursor is put to the start value of the first stage. The value can be updated/modified. Button ‘ENT’ accepts the value and sets the cursor to the next value. When the last value has been reached, ‘ENT’ will cause the system to save the modified preset to system memory and leave the preset configuration screen, returning to the preset selection overview.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | X |  | c | o | n | f | i | g |  |  |  |  |  |
| 2 | S | t | a | r | t | : |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 3 | T | i | m | e | : |  |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 4 | V | o | l | t | : |  |  | 1 | 2 | 3 | 4 | 5 | V |  | 1 | 2 | 3 | 4 | 5 | V |

Where ‘X’ is the number of the currently configurated preset.

| 0-9 | Digit insertion |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Accept inserted value and continue. When last value is reached, ENT will save the modified preset and return to the preset selection overview. |
| X | Leave screen and return to preset config menu without saving |
| others | No function |

When a preset is modified and saved, the system will confirm this action with an additional screen that appears only temporary.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | P | r | e | s | e | t |  | s | a | v | e | d |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This screen will disappear after one second and the preset selection overview will be shown.

#### PID constants

TBD

#### Information and version screen

The full screen on display, containing the system version number, will look like:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | n | o | d | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | S | W |  | V | . |  | 1 | . | 0 | . | 0 | . | 0 |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave screen and return to administrator menu |
| --- | --- |
| others | No function |

### Calibration menu

#### PIN protection

##### PIN insertion screen

The calibration menu is PIN protected. Prior to entering, the administrator must enter the PIN.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | I | n | s | e | r | t |  | P | I | N |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | - | - | - | - |  |  |  |  |  |  |  |  |

| 0-9 | Insert number |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| X | Return to main menu |
| others | No function |

##### PIN accepted

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | P | I | N |  | O | K |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after 3 seconds and the calibration main menu will be entered.

##### PIN denied

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | P | I | N |  | D | E | N | I | E | D |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after 3 seconds and the PIN insertion screen will appear again.

#### Calibration main menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |  |  |
| 2 | > |  | 1 | . | S | e | t | p | o | i | n | t |  | c | h | e | c | k |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ↑ | Move cursor up one line |
| --- | --- |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

The only calibration option now is the verification of setpoint voltages

The following setpoints will be loaded:

| Repair mode | Low setpoint | High setpoint |
| --- | --- | --- |
| Anode | 6000V | 10000V |

The next screen will be the setpoint check menu in both cases.

#### Setpoint check menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | t | p | o | i | n | t |  | c | h | e | c | k |  |  |  |  |  |  |
| 2 | > |  | 1 | . | L | o | w |  | s | e | t | p | o | i | n | t |  |  |  |  |
| 3 |  |  | 2 | . | H | i | g | h |  | s | e | t | p | o | i | n | t |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ↑ | Move cursor up one line |
| --- | --- |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to Cathode/MCP selection menu |
| others | No function |

Danger for unqualified users

Calibration should only be performed by an administrator.

Calibration can only be executed with the lid open. This means that the interlock safety switch should be bypassed. The safety features are now disabled during calibration.

Electrocution hazard

Calibration must be executed while the lid is open. This means the user can access the DUT connections while the high voltage is still present.

##### Low setpoint check screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | L | o | w |  | s | e | t | p | o | i | n | t |  |  |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | - | 6 | 0 | 0 | 0 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 3 | R | o | w | 2 | : |  | - | 6 | 0 | 0 | 0 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 4 | R | o | w | 3 | : |  | - | 6 | 0 | 0 | 0 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |

Where the left value is the low setpoint for the specific selection and the right value is the directly read-back value from the ADC for comparison.

The low setpoint voltage is applied to the outputs automatically as soon as this menu is entered, and a voltage of 0 V is applied to the outputs automatically when this menu is left again.

| X | Leave calibration procedure and return to setpoint check menu |
| --- | --- |
| others | No function |

##### High setpoint check screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | i | g | h |  | s | e | t | p | o | i | n | t |  |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | 1 | 0 | 0 | 0 | 0 | V | ¦ |  | 1 | 2 | 3 | 4 | 5 | V |
| 3 | R | o | w | 2 | : |  | 1 | 0 | 0 | 0 | 0 | V | ¦ |  | 1 | 2 | 3 | 4 | 5 | V |
| 4 | R | o | w | 3 | : |  | 1 | 0 | 0 | 0 | 0 | V | ¦ |  | 1 | 2 | 3 | 4 | 5 | V |

Where the left value is the high setpoint for the specific selection and the right value is the directly read-back value from the ADC for comparison.

The high setpoint voltage is applied to the outputs automatically as soon as this menu is entered, and a voltage of 0 V is applied to the outputs automatically when this menu is left again.

| X | Leave calibration procedure and return to setpoint check menu |
| --- | --- |

## Cathode / MCP drawer

### Start screen

After powering the system, the display will become active and print out a start message, containing the type of repair process and the software version number.

A full start message on display, containing the system version number, will look like:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | a | t | h | o | d | e | / | M | C | P |  | r | e | p | a | i | r |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | S | W |  | V | . |  | 1 | . | 0 | . | 0 | . | 0 |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

There is no user-action needed at this point – the system will switch to the main menu as soon as the system is fully booted and functional.

### Main menu

After booting, the system switches to the main menu.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | a | t | h | o | d | e | / | M | C | P |  | r | e | p | a | i | r |  |  |
| 2 | > |  | 1 | . | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |
| 3 |  |  | 2 | . | A | d | m | i | n | i | s | t | r | a | t | o | r |  |  |  |
| 4 |  |  | 3 | . | C | a | l | i | b | r | a | t | i | o | n |  |  |  |  |  |

Note: The item 2 and 3 are only accessible for the administrator and therefor PIN protected.

| 1-3 | Select item 1-3 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| others | No function |

### Opening the cover

When the system is in main menu, press and hold the 0 key to unlock the cover lock.

### Tube repair menu

Prior to entering the tube repair menu, a selection screen allows to choose between the Cathode and MCP repair process.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | C | a | t | h | o | d | e |  | r | e | p | a | i | r |  |  |
| 3 |  |  | 2 | . | M | C | P |  | r | e | p | a | i | r |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 1-2 | Select item 1-2 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

By choosing either of the two options, the corresponding presets will be loaded. The next screen will be the tube repair menu in both cases.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | S | e | l | e | c | t |  | p | r | e | s | e | t |  |  |  |
| 3 |  |  | 2 | . | S | t | a | r | t |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 1-2 | Select item 1-2 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to Cathode/MCP selection menu |
| others | No function |

#### Select preset menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | l | e | c | t |  | p | r | e | s | e | t |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | P | r | e | s | e | t |  | 1 |  |  |  |  |  |  |  |  |
| 3 |  |  | 2 | . | P | r | e | s | e | t |  | 2 |  |  |  |  |  |  |  |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | 3 |  |  |  |  |  |  |  |  |

There are a total of 9 presets.

| 1-9 | Select item 1-9 respectively and return to tube repair menu |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| → | Go to preset preview menu of the current currently under the cursor |
| ENT | Select preset currently under the cursor and return to tube repair menu |
| X | Leave current menu level and return to tube repair menu |
| others | No function |

Note: The system will always assume preset1 selected when no other selection is done.

#### Preset preview screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | X |  | i | n | f | o |  |  |  |  |  |  |  |
| 2 | S | t | a | r | t | : |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 3 | T | i | m | e | : |  |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 4 | V | o | l | t | : |  |  | - | 1 | 2 | 3 | 4 | m |  | - | 1 | 2 | 3 | 4 | m |

Where ‘X’ is the number of the preset that has been selected for preview.

| ← | Go back select preset menu |
| --- | --- |
| X | Go back select preset menu |
| others | No function |

#### Start repair menu

##### Error messages based on safety

Before starting the repair treatment, it will be verified that the cover of the drawer is closed correctly. For safety reason the cover must be closed before the repair treatment can be started.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  | ! | ! | E | R | R | O | R | ! | ! |  |  |  |  |  |  |
| 2 |  |  |  |  | C | O | V | E | R |  | O | P | E | N |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | H | i | t |  | X |  | t | o |  | c | o | n | t | i | n | u | e |  |

| X | Acknowledge error and return to main menu |
| --- | --- |
| others | No function |

Note: it is not sufficient to close the cover. The system will only react to the X key and return to the main menu. Closing the cover is, however, mandatory before starting a new repair.

##### Repair in progress

When all conditions are met, the repair treatment starts. The display now acts as feedback

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X |  |  | h | h | : | m | m | : | s | s |  | t | o | t | a | l |  |  |  |
| 2 |  |  |  | h | h | : | m | m | : | s | s |  | r | e | m | a | i | n |  |  |
| 3 |  |  | R | 1 |  |  |  |  |  | R | 2 |  |  |  |  |  | R | 3 |  |  |
| 4 | - | 1 | 2 | 3 | 4 | V |  | - | 1 | 2 | 3 | 4 | V |  | - | 1 | 2 | 3 | 4 | V |

The number in row1/column1 represents the selected preset number.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

##### Repair Finished

The standard screen as soon as the repair process finishes looks like

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

| ENT | Return to main menu |
| --- | --- |
| others | No function |

In case that one or more errors occurred, the following screen will be shown indicating the failing row number(s).

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  | E | r | r | o | r |  | i | n |  | r | o | w |  |  |  |  |
| 3 |  |  |  |  |  |  |  | 1 |  | 2 |  | 3 |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

Where the numbers in third row show the row(s) that failed.

| ENT | Return to main menu |
| --- | --- |
| others | No function |

### Repair pause

#### Repair paused

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | R | E | P | A | I | R |  | B | U | S | Y |  |  |  |  |
| 3 |  |  | H | i | t |  |  | X |  |  | t | o |  | P | A | U | S | E |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

When X is hit during an active repair, the screen above appears. This is to suppress accidental keyboard actions. If a pause is requested the user must press X a second time. This screen will remain on display for one second. If the X-button is not pressed within this time, the system will return to the “repair in progress” screen.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

Pausing the repair process automatically turns down the voltage output of all rows to 0 volts. It can take some time to unload all capacities on the rows, so the system will keep waiting until the voltage of all rows is decreased to a safe level. The display will notify about this waiting time with the following screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | W | A | I | T | I | N | G |  | F | O | R |  |  |  |  |  |
| 4 |  |  |  |  |  | P | O | W | E | R |  | D | O | W | N |  |  |  |  |  |

When treatment is paused, the display will change its output to:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | ! | ! | P | A | U | S | E | ! | ! |  |  |  |  |  |  |
| 3 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | e |
| 4 |  | H | i | t |  |  | X |  |  | t | o |  | R | E | S | E | T |  |  |  |

| ENT | Continue the current repair |
| --- | --- |
| X | Reset/quit the treatment and return to the tube repair menu |
| others | No function |

After each pause the remaining repair time increases with 5 minutes.

### Administrator Menu

#### PIN protection

##### PIN insertion screen

The operator menu is PIN protected. Prior to entering, the user must enter the PIN. The default PIN is 0123. It is advised that the administrator changes this PIN as soon as possible.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | I | n | s | e | r | t |  | P | I | N |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | - | - | - | - |  |  |  |  |  |  |  |  |

| 0-9 | Insert number |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| X | Return to main menu |
| others | No function |

##### PIN accepted

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | P | I | N |  | O | K |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the operator menu will be entered.

##### PIN denied

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | P | I | N |  | D | E | N | I | E | D |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after one second and the system will return to the main menu.

#### Administrator main menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | d | m | i | n | i | s | t | r | a | t | i | o | n |  |  |  |  |  |  |
| 2 | > |  | 1 | . | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |
| 3 |  |  | 2 | . | I | / | O |  | c | o | n | t | r | o | l |  |  |  |  |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | c | o | n | f | i | g |  |  |  |

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | . | P | I | D |  | c | o | n | s | t | a | n | t | S |  |  |  |
|  |  |  | 5 | . | I | n | f | o |  | & |  | V | e | r | s | i | o | n |  |  |

| 1-5 | Select item 1-5 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

#### Change PIN menu

##### Insert new Pin

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | N | e | w |  | P | I | N | : |  |  |  | - | - | - | - |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave current menu level and return to administrator menu without changing the PIN |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| 0-9 | Insert number |
| others | No function |

When the 4-digit PIN has been entered, the screen automatically changes to:

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | N | e | w |  | P | I | N | : |  |  |  | * | * | * | * |  |  |
| 4 |  | R | e | p | e | a | t |  | P | I | N | : |  |  |  | - | - | - | - |  |  |

| X | Leave current menu level and return to administrator main menu without changing the PIN |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| 0-9 | Insert number |
| others | No function |

When the 4-digit has been entered a second time, the system will verify that both entered PINs are equal.

##### New Pin OK

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | N | e | w |  | P | I | N |  | a | c | c | e | p | t | e | d |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the administrator main menu will appear again.

##### New Pin failed

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | N | e | w |  | P | I | N |  | d | e | n | i | e | d |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the administrator main menu will appear again.

#### I/O control menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | / | O |  | c | o | n | t | r | o | l |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | R | e | a | d |  | i | n | t | e | r | l | o | c | k |  |  |
| 3 |  |  | 2 | . | S | o | l | e | n | o | i | d | S |  |  |  |  |  |  |  |
| 4 |  |  | 3 | . | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | . | S | e | t |  | v | o | l | t | a | g | e |  | o | u | t |  |

| 1-4 | Select item 1-4 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to administrator menu |
| others | No function |

##### Read interlock

Reading the interlock status is a constant action, meaning that the screen will immediately update its content with the interlock status.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | C | o | v | e | r |  | i | n | t | e | r | l | o | c | k |  | i | s |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

If interlock is open:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  |  | o | p | e | n |  |  |  |  |  |  |  |  |

If interlock is closed:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  | c | l | o | s | e | d |  |  |  |  |  |  |  |

| X | Leave current menu level and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Solenoids

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | o | l | e | n | o | i | d | s |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | H | o | l | d |  | 0 |  | t | o |  | t | o | g | g | l | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 0 | Press: activate solenoids (open cover) Release: de-activate solenoids (seal cover) |
| --- | --- |
| X | Leave current menu level and return to administrator I/O menu |
| others | No function |

##### Get voltage in

The screen to read the voltage input is a read-only screen, meaning that no user action is accepted (other than leaving the menu).

The screen shows the currently read voltage on all three rows in both voltage (left value) and the corresponding hexadecimal value of the DA converter.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | A | B | C | D | x |  |
| 3 | R | o | w | 2 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | A | B | C | D | x |  |
| 4 | R | o | w | 3 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | A | B | C | D | x |  |

| X | Leave screen and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Set voltage out

The screen to set the voltage output enables the administrator to apply a voltage to the three rows.

The cursor is put at the first row when entering the menu.

The screen shows two values per row. The left value is the value that the administrator inserts and applies to the DA converter of that specific row. The right value of that row is the actual value that is read back from the AD converter of that same row.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 3 | R | o | w | 2 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 4 | R | o | w | 3 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |

| 0-9 | Used for value insertion |
| --- | --- |
| ↑ | Move cursor up one row |
| ↓ | Move cursor down one row |
| ← | Backspace – delete last entered digit |
| ENT | Accept inserted value and apply to DA converter of that row |
| X | Leave screen and return to administrator I/O menu |
| others | No function |

Electrocution hazard

Set voltage out can be executed while the lid is open. This means the user can access the DUT connections while the high voltage is still present.

Leaving the set voltage out menu causes the following screen to appear.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | A | l | l |  | r | o | w | s |  | r | e | s | e | t |  | t | o |  | 0 | V |
| 4 |  | P | r | e | s | s |  | X |  | t | o |  | c | o | n | t | i | n | u | e |

| X | Leave screen and return to administrator I/O menu |
| --- | --- |
| others | No function |

#### Preset configuration

The repair process follows a configurable preset. There are two types of presets, a one-stage and a two-stage repair preset. See the following images for an explanation of the presets.

Figure 3: One-stage repair preset

Figure 4: Two-stage repair preset

| Parameter | Description |
| --- | --- |
| Start | Ramp-up time (minutes) |
| Time | Repair duration (minutes) |
| Volt | Repair voltage (Volt) |

Select a preset to configure.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | C | a | t | h | o | d | e |  | r | e | p | a | i | r |  |  |
| 3 |  |  | 2 | . | M | C | P |  | r | e | p | a | i | r |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 1-2 | Select item 1-2 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to administrator menu |
| others | No function |

By choosing either of the two options, the corresponding presets will be loaded. The next screen will be the preset configuration menu in both cases.

Select a preset to configure.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | c | o | n | f | i | g |  |  |  |  |  |  |  |
| 2 |  |  | 1 | . | P | r | e | s | e | t |  | 1 |  | c | o | n | f | i | g |  |
| 3 |  |  | 2 | . | P | r | e | s | e | t |  | 2 |  | c | o | n | f | i | g |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | 3 |  | c | o | n | f | i | g |  |

The screen continues until item “9.Preset 9 config”.

| 1-9 | Select item 1-9 respectively |
| --- | --- |
| ↑ | Move cursor up one row |
| ↓ | Move cursor down one row |
| ENT | Select current preset under the cursor |
| X | Leave screen and return to Cathode/MCP selection menu |
| others | No function |

The first six presets only contain one stage, while preset seven, eight and nine contain two repair stages. The screen underneath represents a preset with two stages. For a single-stage preset, the second column with values is not available.

When entering this screen, the cursor is put to the start value of the first stage. The value can be updated/modified. Button ‘ENT’ accepts the value and sets the cursor to the next value. When the last value has been reached, ‘ENT’ will cause the system to save the modified preset to system memory and leave the preset configuration screen, returning to the preset selection overview.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | X |  | c | o | n | f | i | g |  |  |  |  |  |
| 2 | S | t | a | r | t | : |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 3 | T | i | m | e | : |  |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 4 | V | o | l | t | : |  |  | - | 1 | 2 | 3 | 4 | V |  | - | 1 | 2 | 3 | 4 | V |

Where ‘X’ is the number of the currently configurated preset.

| 0-9 | Digit insertion |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Accept inserted value and continue. When last value is reached, ENT will save the modified preset and return to the preset selection overview. |
| X | Leave screen and return to preset config menu without saving |
| others | No function |

When a preset is modified and saved, the system will confirm this action with an additional screen that appears only temporary.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | P | r | e | s | e | t |  | s | a | v | e | d |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This screen will disappear after one second and the preset selection overview will be shown.

#### PID Constants

TBD

#### Information and version screen

The full screen on display, containing the system version number, will look like:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | a | t | h | o | d | e | / | M | C | P |  | r | e | p | a | i | r |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | S | W |  | V | . |  | 1 | . | 0 | . | 0 | . | 0 |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave screen and return to administrator menu |
| --- | --- |
| others | No function |

### Calibration menu

#### PIN protection

##### PIN insertion screen

The calibration menu is PIN protected. Prior to entering, the administrator must enter the PIN.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | I | n | s | e | r | t |  | P | I | N |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | - | - | - | - |  |  |  |  |  |  |  |  |

| 0-9 | Insert number |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| X | Return to main menu |
| others | No function |

##### PIN accepted

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | P | I | N |  | O | K |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after 3 seconds and the calibration main menu will be entered.

##### PIN denied

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | P | I | N |  | D | E | N | I | E | D |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after 3 seconds and the PIN insertion screen will appear again.

#### Calibration main menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |  |  |
| 2 | > |  | 1 | . | S | e | t | p | o | i | n | t |  | c | h | e | c | k |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ↑ | Move cursor up one line (currently not implemented) |
| --- | --- |
| ↓ | Move cursor down one line (currently not implemented) |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

The only calibration option now is the verification of setpoint voltages

#### Cathode/MCP selection menu

Prior to entering the setpoint check menu, a selection screen allows to differ between cathode and MCP.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | C | a | t | h | o | d | e |  | r | e | p | a | i | r |  |  |
| 3 |  |  | 2 | . | M | C | P |  | r | e | p | a | i | r |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 1-2 | Select item 1-2 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to calibration menu |
| others | No function |

By choosing either of the two options, the corresponding setpoints will be loaded, which are:

| Repair mode | Low setpoint | High setpoint |
| --- | --- | --- |
| Cathode | -200 V | -1000 V |
| MCP | -500 V | -2000 V |

The next screen will be the setpoint check menu in both cases.

#### Setpoint check menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | t | p | o | i | n | t |  | c | h | e | c | k |  |  |  |  |  |  |
| 2 | > |  | 1 | . | L | o | w |  | s | e | t | p | o | i | n | t |  |  |  |  |
| 3 |  |  | 2 | . | H | i | g | h |  | s | e | t | p | o | i | n | t |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ↑ | Move cursor up one line |
| --- | --- |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to Cathode/MCP selection menu |
| others | No function |

Danger for unqualified users

Calibration should only be performed by an administrator.

Calibration can only be executed with the lid open. This means that the interlock safety switch should be bypassed. The safety features are now disabled during calibration.

Electrocution hazard

Calibration must be executed while the lid is open. This means the user can access the DUT connections while the high voltage is still present.

##### Low setpoint check screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | L | o | w |  | s | e | t | p | o | i | n | t |  |  |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 3 | R | o | w | 2 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 4 | R | o | w | 3 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |

Where the left value is the low setpoint for the specific selection and the right value is the directly read-back value from the ADC for comparison.

The low setpoint voltage is applied to the outputs automatically as soon as this menu is entered, and a voltage of 0 V is applied to the outputs automatically when this menu is left again.

| X | Leave calibration procedure and return to setpoint check menu |
| --- | --- |
| others | No function |

##### High setpoint check screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | i | g | h |  | s | e | t | p | o | i | n | t |  |  |  |  |  |  |  |
| 2 | R | o | w | 1 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 3 | R | o | w | 2 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |
| 4 | R | o | w | 3 | : |  | - | 1 | 2 | 3 | 4 | V | ¦ |  | - | 1 | 2 | 3 | 4 | V |

Where the left value is the high setpoint for the specific selection and the right value is the directly read-back value from the ADC for comparison.

The high setpoint voltage is applied to the outputs automatically as soon as this menu is entered, and a voltage of 0 V is applied to the outputs automatically when this menu is left again.

| X | Leave calibration procedure and return to setpoint check menu |
| --- | --- |
| others | No function |

## Tesla drawer

### Start screen

After powering the system, the display will become active and print out a start message, containing the type of repair process and the software version number.

A full start message on display, containing the system version number, will look like:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | e | s | l | a |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | S | W |  | V | . |  | 1 | . | 0 | . | 0 | . | 0 |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

There is no user-action needed at this point – the system will switch to the main menu as soon as the system is fully booted and functional.

### Main menu

After booting, the system switches to the main menu.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | e | s | l | a |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |
| 3 |  |  | 2 | . | A | d | m | i | n | i | s | t | r | a | t | o | r |  |  |  |
| 4 |  |  | 3 | . | C | a | l | i | b | r | a | t | i | o | n |  |  |  |  |  |

Note: The item 2 and 3 are only accessible for the administrator and therefor PIN protected.

| 1-3 | Select item 1-3 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| others | No function |

### Opening the cover

When the system is in main menu, press and hold the 0 key to unlock the cover lock.

### Tube repair menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | u | b | e |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | S | e | l | e | c | t |  | p | r | e | s | e | t |  |  |  |
| 3 |  |  | 2 | . | S | t | a | r | t |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 1-2 | Select item 1-2 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

#### Select preset menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | l | e | c | t |  | p | r | e | s | e | t |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | P | r | e | s | e | t |  | 1 |  |  |  |  |  |  |  |  |
| 3 |  |  | 2 | . | P | r | e | s | e | t |  | 2 |  |  |  |  |  |  |  |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | 3 |  |  |  |  |  |  |  |  |

The screen continues to item “9.Preset 9”.

| 1-9 | Select item 1-9 respectively and return to tube repair menu |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| → | Go to preset preview menu of the current currently under the cursor |
| ENT | Select preset currently under the cursor and return to tube repair menu |
| X | Leave current menu level and return to tube repair menu |
| others | No function |

Note: The system will always assume preset1 selected when no other selection is done.

#### Preset preview screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | X |  | i | n | f | o |  |  |  |  |  |  |  |
| 2 | S | t | a | r | t | : |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 3 | T | i | m | e | : |  |  |  |  | 1 | 2 | 3 | m |  |  |  | 1 | 2 | 3 | m |
| 4 | V | o | l | t | : |  |  | 1 | 2 | 3 | 4 | 5 | m |  | 1 | 2 | 3 | 4 | 5 | m |

Where ‘X’ is the number of the preset that has been selected for preview.

| ← | Go back select preset menu |
| --- | --- |
| X | Go back select preset menu |
| others | No function |

#### Start repair menu

##### Error messages based on safety

Before starting the repair treatment, it will be verified that the cover of the drawer is closed correctly. For safety reason the cover must be closed before the repair treatment can be started.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  | ! | ! | E | R | R | O | R | ! | ! |  |  |  |  |  |  |
| 2 |  |  |  |  | C | O | V | E | R |  | O | P | E | N |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | H | i | t |  | X |  | t | o |  | c | o | n | t | i | n | u | e |  |

| X | Acknowledge error and return to main menu |
| --- | --- |
| others | No function |

Note: it is not sufficient to close the cover. The system will only react to the X key and return to the main menu. Closing the cover is, however, mandatory before starting a new repair.

##### Repair in progress

When all conditions are met, the repair treatment starts. The display now acts as feedback

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | X |  |  | h | h | : | m | m | : | s | s |  | t | o | t | a | l |  |  |  |
| 2 |  |  |  | h | h | : | m | m | : | s | s |  | r | e | m | a | i | n |  |  |
| 3 |  |  |  |  |  |  |  |  |  | R | 2 |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | 1 | 2 | 3 | 4 | V |  |  |  |  |  |  |  |

The number in row1/column1 represents the selected preset number.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

##### Repair finished

The standard screen as soon as the repair process finishes looks like

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

| ENT | Return to main menu |
| --- | --- |
| others | No function |

In case that one or more errors occurred, the following screen will be shown indicating the failing row number(s).

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  | E | r | r | o | r |  | i | n |  | r | o | w |  |  |  |  |
| 3 |  |  |  |  |  |  |  | 1 |  | 2 |  | 3 |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

Where the numbers in third row show the row(s) that failed.

| ENT | Return to main menu |
| --- | --- |
| others | No function |

### Repair pause

#### Repair paused

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | R | E | P | A | I | R |  | B | U | S | Y |  |  |  |  |
| 3 |  |  | H | i | t |  |  | X |  |  | t | o |  | P | A | U | S | E |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

When X is hit during an active repair, the screen above appears. This is to suppress accidental keyboard actions. If a pause is requested the user must press X a second time. This screen will remain on display for one second. If the X-button is not pressed within this time, the system will return to the “repair in progress” screen.

| X | Pause the current repair action |
| --- | --- |
| others | No function |

Pausing the repair process automatically turns down the voltage output of all rows to 0 volts. It can take some time to unload all capacities on the rows, so the system will keep waiting until the voltage of all rows is decreased to a safe level. The display will notify about this waiting time with the following screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | W | A | I | T | I | N | G |  | F | O | R |  |  |  |  |  |
| 4 |  |  |  |  |  | P | O | W | E | R |  | D | O | W | N |  |  |  |  |  |

When treatment is paused, the display will change its output to:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  | ! | ! | P | A | U | S | E | ! | ! |  |  |  |  |  |  |
| 3 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | e |
| 4 |  | H | i | t |  |  | X |  |  | t | o |  | R | E | S | E | T |  |  |  |

| ENT | Continue the current repair |
| --- | --- |
| X | Reset/quit the treatment and return to the tube repair menu |
| others | No function |

After each pause the remaining repair time increases with 5 minutes.

### Administrator Menu

#### PIN protection

##### PIN insertion screen

The operator menu is PIN protected. Prior to entering, the user must enter the PIN. The default PIN is 0123. It is advised that the administrator changes this PIN as soon as possible.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | I | n | s | e | r | t |  | P | I | N |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | - | - | - | - |  |  |  |  |  |  |  |  |

| 0-9 | Insert number |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| X | Return to main menu |
| others | No function |

##### PIN accepted

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | P | I | N |  | O | K |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the operator menu will be entered.

##### PIN denied

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | P | I | N |  | D | E | N | I | E | D |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after one second and the system will return to the main menu.

#### Administrator main menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | A | d | m | i | n | i | s | t | r | a | t | i | o | n |  |  |  |  |  |  |
| 2 | > |  | 1 | . | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |
| 3 |  |  | 2 | . | I | / | O |  | c | o | n | t | r | o | l |  |  |  |  |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | c | o | n | f | i | g |  |  |  |

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | . | P | I | D |  | c | o | n | s | t | a | n | t | S |  |  |  |
|  |  |  | 5 | . | I | n | f | o |  | & |  | V | e | r | s | i | o | n |  |  |

| 1-5 | Select item 1-5 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

#### Change PIN menu

##### Insert new Pin

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | N | e | w |  | P | I | N | : |  |  |  | - | - | - | - |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave current menu level and return to administrator menu without changing the PIN |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| 0-9 | Insert number |
| others | No function |

When the 4-digit PIN has been entered, the screen automatically changes to:

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | h | a | n | g | e |  | P | I | N |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | N | e | w |  | P | I | N | : |  |  |  | * | * | * | * |  |  |
| 4 |  | R | e | p | e | a | t |  | P | I | N | : |  |  |  | - | - | - | - |  |  |

| X | Leave current menu level and return to administrator main menu without changing the PIN |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| 0-9 | Insert number |
| others | No function |

When the 4-digit has been entered a second time, the system will verify that both entered PINs are equal.

##### New Pin OK

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  | N | e | w |  | P | I | N |  | a | c | c | e | p | t | e | d |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the administrator main menu will appear again.

##### New Pin failed

| / |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | N | e | w |  | P | I | N |  | d | e | n | i | e | d |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after three seconds and the administrator main menu will appear again.

#### I/O control menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | I | / | O |  | c | o | n | t | r | o | l |  |  |  |  |  |  |  |  |  |
| 2 | > |  | 1 | . | R | e | a | d |  | i | n | t | e | r | l | o | c | k |  |  |
| 3 |  |  | 2 | . | S | o | l | e | n | o | i | d | S |  |  |  |  |  |  |  |
| 4 |  |  | 3 | . | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4 | . | S | e | t |  | v | o | l | t | a | g | e |  | o | u | t |  |

| 1-4 | Select item 1-4 respectively |
| --- | --- |
| ↑ | Move cursor up one line |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to administrator menu |
| others | No function |

##### Read interlock

Reading the interlock status is a constant action, meaning that the screen will immediately update its content with the interlock status.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | C | o | v | e | r |  | i | n | t | e | r | l | o | c | k |  | i | s |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

If interlock is open:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  |  | o | p | e | n |  |  |  |  |  |  |  |  |

If interlock is closed:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  | c | l | o | s | e | d |  |  |  |  |  |  |  |

| X | Leave current menu level and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Solenoids

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | o | l | e | n | o | i | d | s |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  | H | o | l | d |  | 0 |  | t | o |  | t | o | g | g | l | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 0 | Press: activate solenoids (open cover) Release: de-activate solenoids (seal cover) |
| --- | --- |
| X | Leave current menu level and return to administrator I/O menu |
| others | No function |

##### Get voltage in

The screen to read the voltage input is a read-only screen, meaning that no user action is accepted (other than leaving the menu).

The screen shows the currently read voltage on all three rows in both voltage (left value) and the corresponding hexadecimal value of the DA converter.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | G | e | t |  | v | o | l | t | a | g | e |  | i | n |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | R | o | w | 2 | : |  |  | 1 | 2 | 3 | 4 | V | ¦ |  | A | B | C | D | x |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave screen and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Set voltage out

The screen to set the voltage input is allows to apply a voltage to the three rows.

The screen shows two values per row. The left value is the value that the administrator inserts and applies to the DA converter of that specific row. The right value of that row is the actual value that is read back from the AD converter of that same row.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | t |  | v | o | l | t | a | g | e |  | o | u | t |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | R | o | w | 2 | : |  |  | 1 | 2 | 3 | 4 | V | ¦ |  |  | 1 | 2 | 3 | 4 | V |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 0-9 | Used for value insertion |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Accept inserted value and apply to DA converter of that row |
| X | Leave screen and return to administrator I/O menu |
| others | No function |

Electrocution hazard

Set voltage out can be executed while the lid is open. This means the user can access the DUT connections while the high voltage is still present.

Leaving the set voltage out menu causes the following screen to appear.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | A | l | l |  | r | o | w | s |  | r | e | s | e | t |  | t | o |  | 0 | V |
| 4 |  | P | r | e | s | s |  | X |  | t | o |  | c | o | n | t | i | n | u | e |

| X | Leave screen and return to administrator I/O menu |
| --- | --- |
| others | No function |

##### Teslagun relais

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | e | s | l | a | g | u | n |  | r | e | l | a | i | s |  |  |  |  |  |
| 2 |  |  | H | o | l | d |  | 0 |  | t | o |  | t | o | g | g | l | e |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| 0 | Press: activate relais Release: de-activate relais |
| --- | --- |
| X | Leave current menu level and return to administrator I/O menu |
| others | No function |

#### Preset configuration

The repair process follows a configurable one-stage preset. See the following image for an explanation of the preset.

Figure 5: One stage repair preset

| Parameter | Description |
| --- | --- |
| Start | Ramp-up time (minutes) |
| Time | Repair duration (minutes) |
| Volt | Repair voltage (Volt) |

Select a preset to configure.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | c | o | n | f | i | g |  |  |  |  |  |  |  |
| 2 |  |  | 1 | . | P | r | e | s | e | t |  | 1 |  | c | o | n | f | i | g |  |
| 3 |  |  | 2 | . | P | r | e | s | e | t |  | 2 |  | c | o | n | f | i | g |  |
| 4 |  |  | 3 | . | P | r | e | s | e | t |  | 3 |  | c | o | n | f | i | g |  |

The screen continues until item “9.Preset 9 config”.

| 1-9 | Select item 1-9 respectively |
| --- | --- |
| ↑ | Move cursor up one row |
| ↓ | Move cursor down one row |
| ENT | Select current preset under the cursor |
| X | Leave screen and return to Cathode/MCP selection menu |
| others | No function |

The screen underneath represents a preset with one stage.

When entering this screen, the cursor is put to the start value of the first stage. The value can be updated/modified. Button ‘ENT’ accepts the value and sets the cursor to the next value. When the last value has been reached, ‘ENT’ will cause the system to save the modified preset to system memory and leave the preset configuration screen, returning to the preset selection overview.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | r | e | s | e | t |  | X |  | c | o | n | f | i | g |  |  |  |  |  |
| 2 | S | t | a | r | t | : |  |  |  | 1 | 2 | 3 | m |  |  |  |  |  |  |  |
| 3 | T | i | m | e | : |  |  |  |  | 1 | 2 | 3 | m |  |  |  |  |  |  |  |
| 4 | V | o | l | t | : |  |  |  | 1 | 2 | 3 | 4 | V |  |  |  |  |  |  |  |

Where ‘X’ is the number of the currently configurated preset.

| 0-9 | Digit insertion |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Accept inserted value and continue. When last value is reached, ENT will save the modified preset and return to the preset selection overview. |
| X | Leave screen and return to preset config menu without saving |
| others | No function |

When a preset is modified and saved, the system will confirm this action with an additional screen that appears only temporary.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | P | r | e | s | e | t |  | s | a | v | e | d |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

This screen will disappear after one second and the preset selection overview will be shown.

#### PID constraints

TBD

#### Information and version screen

The full screen on display, containing the system version number, will look like:

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | T | e | s | l | a |  | r | e | p | a | i | r |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  | S | W |  | V | . |  | 1 | . | 0 | . | 0 | . | 0 |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| X | Leave screen and return to administrator menu |
| --- | --- |
| others | No function |

### Calibration menu

#### PIN protection

##### PIN insertion screen

The calibration menu is PIN protected. Prior to entering, the user must enter the PIN.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | I | n | s | e | r | t |  | P | I | N |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  | - | - | - | - |  |  |  |  |  |  |  |  |

| 0-9 | Insert number |
| --- | --- |
| ← | Backspace – delete last entered digit |
| ENT | Confirm inserted PIN |
| X | Return to main menu |
| others | No function |

##### PIN accepted

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  | P | I | N |  | O | K |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after 3 seconds and the calibration main menu will be entered.

##### PIN denied

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  | P | I | N |  | D | E | N | I | E | D |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| others | No function |
| --- | --- |

No action required. The screen will be left after 3 seconds and the PIN insertion screen will appear again.

#### Calibration main menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | C | a | l | i | b | r | a | t | i | o | n |  | m | e | n | u |  |  |  |  |
| 2 | > |  | 1 | . | S | e | t | p | o | i | n | t |  | c | h | e | c | k |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ↑ | Move cursor up one line (currently not implemented) |
| --- | --- |
| ↓ | Move cursor down one line (currently not implemented) |
| ENT | Select currently current item |
| X | Leave current menu level and return to main menu |
| others | No function |

The only calibration option now is the verification of setpoint voltages

##### Setpoint check menu

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | S | e | t | p | o | i | n | t |  | c | h | e | c | k |  |  |  |  |  |  |
| 2 | > |  | 1 | . | L | o | w |  | s | e | t | p | o | i | n | t |  |  |  |  |
| 3 |  |  | 2 | . | H | i | g | h |  | s | e | t | p | o | i | n | t |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| ↑ | Move cursor up one line |
| --- | --- |
| ↓ | Move cursor down one line |
| ENT | Select currently current item |
| X | Leave current menu level and return to Cathode/MCP selection menu |
| others | No function |

Danger for unqualified users

Calibration should only be performed by an administrator.

Calibration can only be executed with the lid open. This means that the interlock safety switch should be bypassed. The safety features are now disabled during calibration.

Electrocution hazard

Calibration must be executed while the lid is open. This means the user can access the DUT connections while the high voltage is still present.

###### Low setpoint check screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | L | o | w |  | s | e | t | p | o | i | n | t |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | R | o | w | 2 | : |  |  | 3 | 0 | 0 | 0 | V | ¦ |  |  | 1 | 2 | 3 | 4 | V |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Where the left value is the low setpoint and the right value is the directly read-back value from the ADC for comparison.

The low setpoint voltage is applied to the outputs automatically as soon as this menu is entered, and a voltage of 0 V is applied to the outputs automatically when this menu is left again.

| X | Leave calibration procedure and return to setpoint check menu |
| --- | --- |
| others | No function |

###### High setpoint check screen

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | H | i | g | h |  | s | e | t | p | o | i | n | t |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 | R | o | w | 2 | : |  |  | 6 | 0 | 0 | 0 | V | ¦ |  |  | 1 | 2 | 3 | 4 | V |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Where the left value is the high setpoint and the right value is the directly read-back value from the ADC for comparison.

The high setpoint voltage is applied to the outputs automatically as soon as this menu is entered, and a voltage of 0 V is applied to the outputs automatically when this menu is left again.

| X | Leave calibration procedure and return to setpoint check menu |
| --- | --- |
| others | No function |

## Error messages

### Cover open

When the cover is not closed properly when the repair process is started, the following screen will be shown.

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  | ! | ! | E | R | R | O | R | ! | ! |  |  |  |  |  |  |
| 2 |  |  |  |  | C | O | V | E | R |  | O | P | E | N |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  | H | i | t |  | X |  | t | o |  | c | o | n | t | i | n | u | e |  |

| X | Acknowledge error and return to main menu |
| --- | --- |
| others | No function |

Ensure that the cover is properly closed and both solenoids hold the cover in the closed position. Press the X key to return to the main menu and start the repair process again.

### Error in row

When an error occurred during a repair process, the following screen will be shown after the repair has finished, indicating the failing row number(s).

| / | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | R | E | P | A | I | R |  | F | I | N | I | S | H | E | D |  |  |
| 2 |  |  |  |  | E | r | r | o | r |  | i | n |  | r | o | w |  |  |  |  |
| 3 |  |  |  |  |  |  |  | 1 |  | 2 |  | 3 |  |  |  |  |  |  |  |  |
| 4 |  | H | i | t |  | E | N | T |  | t | o |  | c | o | n | t | i | n | u | E |

Where the numbers in third row show the row(s) that failed.

| ENT | Return to main menu |
| --- | --- |
| others | No function |

# Contact and support

## Maintenance and cleaning

The device needs no maintenance.

Cleaning:

- Disconnect the instrument from the mains net.

- Clean the external surfaces of the instrument with a soft cloth dampened with mild detergent or water.

## Repairs

If the equipment suddenly performs in an unexpected way, which indicates a fault, or it has an obvious defect, this cannot and must not be repaired by the user. Contact the supplier in case of suspicion and elicit the steps to be taken.

It will then usually be necessary to return the device to Photonis. If a return for checking or repair is to be carried out, ensure that:

- The supplier has been contacted and it is clarified how and where the equipment should be sent.

- The device is in fully assembled state and in suitable transport packaging.

- A fault description in as much detail as possible is attached.

- If shipping destination is abroad, the necessary customs papers are attached.

## Replacing a defect mains fuse

The device is protected by a fuse which is inside a fuse holder on the rear of the device. The fuse rating is printed on the identification label. Replace the fuse only with one of same size and rating.

## Contact

Questions or problems with operation of the device, use of optional components, with the documentation or software, can be addressed to technical support either by telephone or e-Mail.

| Adress | e-Mail | Telephone |
| --- | --- | --- |
| Photonis Netherlands BV Dwazziewegen 2 9301 ZR Roden The Netherlands | Technical support: All other topics: | Switchboard: +31 50 501 8808 Support: |